MCT333s (UG2018) - Mechatronic Systems Design

TEAM 2

APEX

| Nouran Mohamed Hassan | 2200122 |
| --- | --- |
| Heba Rabea Teleb | 2200428 |
| Mohammed Ibrahim | 2200286 |
| Malak sherif | 2200376 |
| Fady Ramy Adel | 2200811 |
| Hanan Mohamed Soliman | 2200391 |

Table Of Contents

1. Software and Vision System Design	2

1.1 System Overview	2

1.2 Hardware Platform	2

1.3 Wi-Fi Setup	3

1.4 Camera Initialization	3

1.5 QR Code Detection Algorithm	4

1.6 Real-Time Task Architecture	4

1.7 Wireless Communication	5

Table Of Figures

Figure 1: Scanned QR 1	6

Figure 2 :Scanned QR 2	6

# 1. Software and Vision System Design

## 1.1 System Overview

The perception subsystem of the robot is responsible for detecting and decoding QR codes placed on the competition cubes. The decoded data indicates the target color of the box, which determines the correct delivery location.

According to the competition rules, each cube contains a QR code encoding the color (Red, Blue, or Green) which must be read by the robot before the sorting and delivery process.

To implement this functionality, an ESP32-S3 Cam module with an integrated GC2145 2-MP camera sensor was used. The ESP32 handles:

- Image acquisition

- QR code detection

- Wireless streaming

- Communication with the operator interface

The system processes camera frames in real time and decodes QR codes using the Quirc QR-code recognition library.

## 1.2 Hardware Platform

The vision system uses:

Microcontroller

- ESP32-S3 CAM

Camera Sensor

- 2 MP GC2145

CPU

- Dual-core 32-bit ESP32-S3

RAM

- 520 KB internal + 8 MB PSRAM

Communication

- WiFi Access Point mode

The external PSRAM is particularly important because it allows buffering multiple camera frames and improves streaming performance.

## 1.3 Wi-Fi Setup

ESP32-CAM operates in Access Point (AP) mode.

Users connect via mobile or PC to the AP (SSID: ESPCAM, Password: 12345678).

Two web endpoints are served:

- Port 80: Web dashboard with QR code results and control.

- Port 81: Raw video streaming for live monitoring.

## 1.4 Camera Initialization

The ESP32 camera is configured to capture grayscale frames at QVGA resolution (320×240).

This configuration is chosen because:

- Lower resolution increases processing speed

- Grayscale reduces memory usage

- Improves QR detection reliability

Code  – Camera Configuration

If PSRAM is detected, the system allocates two frame buffers to increase performance.

## 1.5 QR Code Detection Algorithm

QR decoding is implemented using the Quirc library, which is optimized for embedded systems.

The algorithm works as follows:

- Capture camera frame

- Resize QR decoder buffer

- Copy frame into the decoder

- Detect QR patterns

- Decode payload

## 1.6 Real-Time Task Architecture

The system uses FreeRTOS multitasking, which allows the ESP32 to run multiple operations simultaneously.

Two main tasks run in parallel:

Task 1 – QR Detection Task

Runs on Core 0.

Responsibilities:

- Capture frames

- Decode QR codes

- Store the detected result

Task 2 – Streaming Task

Runs in the main loop.

Responsibilities:

- Send live video to the browser

- Provide monitoring interface

## 1.7 Wireless Communication

The ESP32 operates in Access Point (AP) mode.

Users connect directly to the robot and open the web dashboard.

Dashboard Features

- Live camera feed

- QR detection results

- FPS monitoring

Figure 1: Scanned QR 1

Figure 2:Scanned QR 2